Illicit Discharge Detection and Elimination (IDDE) Plan

##MUNICIPALITY

Prepared By:

Seacoast Stormwater Coalition &

Manchester/Nashua Stormwater Coalition

Permit Year 1

EPA NPDES Permit Number NHR041000

Table of Contents

Illicit Discharge Detection and Elimination Plan

1	IDDE Program Implementation Timeline	3

1.1	MS4 Program ………………………………………………………………………..3

1.2	Illicit Discharges ……………………………………………………………………..3

1.3	Allowable Non-Stormwater Discharges ……………………………………………...4

2	Authority and Statement of IDDE Responsibilities	6

2.1	Legal Authority	6

2.2	Statement of Responsibilities	7

3	Stormwater System Mapping	8

3.1	Phase I Mapping…………………………………………………………………....8

3.2	Phase II Mapping ……………………………………………………………..……8

4	Sanitary Sewer Overflows (SSOs)	9

5	Assessment and Priority Ranking of Outfalls	10

5.1	Outfall Catchment Delineations	10

5.2	Outfall and Interconnection Inventory and Initial Ranking	10

6	Dry Weather Outfall Screening and Sampling	13

6.1	Dry Weather Screening/Sampling Procedure …………………………….…….13

6.1.1	General Procedure ……………………………………………….……....13

6.1.2	Field Equipment - Dry Weather Screening & Sampling ……………....14

6.1.3	Sample Collection and Analysis …………………………………………15

6.2	Ranking of Outfalls and Interconnections …………………………...………….15

7	Catchment Investigations	17

7.1	Illicit Discharge Removal	17

8	Training	18

9	Progress Reporting	18

Tables

Table 1-1. IDDE Program Implementation Timeline	5

Table 6-1. Field Equipment – Dry Weather Outfall Screening and Sampling	15

Table 9-1. SSO Inventory	22

Figures

Figure 1-1. IDDE Investigation Procedure Framework	5

Appendices

Appendix A – Legal Authority (IDDE Bylaw or Ordinance)

Appendix B – List of Impaired Waters, Storm System Mapping & SSO Inventory

Appendix C – Outfall Inventory and Priority Ranking Matrix

Appendix D - Field Forms, Sample Bottle Labels, and Chain of Custody Forms

Appendix E – Water Quality Analysis Instructions, User’s Manuals and Standard Operating Procedures

Appendix F – IDDE Employee Training Record

Appendix G – Source Isolation and Confirmation Methods: Instructions, Manuals, and SOPs

# IDDE Program Implementation Timeline

## MS4 Program

This Illicit Discharge Detection and Elimination (IDDE) Plan has been developed by ##MUNICIPALITY to address the requirements of the United States Environmental Protection Agency’s (USEPA’s) 2017 National Pollutant Discharge Elimination System (NPDES) General Permit for Stormwater Discharges from Small Municipal Separate Storm Sewer Systems (MS4) in New Hampshire, hereafter referred to as the “2017 New Hampshire MS4 Permit” or “MS4 Permit.”

The 2017 New Hampshire MS4 Permit requires that each permittee, or regulated community, address six Minimum Control Measures.  These measures include the following:

- Public Education and Outreach

- Public Involvement and Participation

- Illicit Discharge Detection and Elimination Program

- Construction Site Stormwater Runoff Control

- Stormwater Management in New Development and Redevelopment (Post Construction Stormwater Management); and

- Good Housekeeping and Pollution Prevention for Permittee Owned Operations.

Under Minimum Control Measure 3, the permittee is required to implement an IDDE program to systematically find and eliminate sources of non-stormwater discharges to its municipal separate storm sewer system and implement procedures to prevent such discharges. The IDDE program must also be recorded in a written (hardcopy or electronic) document. This IDDE Plan has been prepared to address this requirement.

## Illicit Discharges

An “illicit discharge” is any discharge to a drainage system that is not composed entirely of stormwater, with the exception of discharges pursuant to a NPDES permit (other than the NPDES permit for discharges from the MS4) and discharges resulting from fire-fighting activities.

Illicit discharges may take a variety of forms. Illicit discharges may enter the drainage system through direct or indirect connections. Direct connections may be relatively obvious, such as cross-connections of sewer services to the storm drain system. Indirect illicit discharges may be more difficult to detect or address, such as failing septic systems that discharge untreated sewage to a ditch within the MS4, or a sump pump that discharges contaminated water on an intermittent basis.

Some illicit discharges are intentional, such as dumping used oil (or other pollutant) into catch basins, a resident or contractor illegally tapping a new sewer lateral into a storm drain pipe to avoid the costs of a sewer connection fee and service, and illegal dumping of yard wastes into surface waters.

Some illicit discharges are related to the unsuitability of original infrastructure to the modern regulatory environment. Examples of illicit discharges in this category include connected floor drains in old buildings, as well as sanitary sewer overflows that enter the drainage system. Sump pumps legally connected to the storm drain system may be used inappropriately, such as for the disposal of floor washwater or old household products, in many cases due to a lack of understanding on the part of the homeowner.

Elimination of some discharges may require substantial costs and efforts, such as funding and designing a project to reconnect sanitary sewer laterals. Others, such as improving self-policing of dog waste management, can be accomplished by outreach in conjunction with the minimal additional cost of dog waste bins and the municipal commitment to disposal of collected materials on a regular basis.

Regardless of the intention, when not addressed, illicit discharges can contribute high levels of pollutants, such as heavy metals, toxics, oil, grease, solvents, nutrients, and pathogens to surface waters.

## Allowable Non-Stormwater Discharges

The following categories of non-storm water discharges are allowed under the MS4 Permit unless the permittee, USEPA identifies any category or individual discharge of non-stormwater discharge as a significant contributor of pollutants to the MS4:

- Water line flushing

- Landscape irrigation

- Diverted stream flows

- Uncontaminated ground water infiltration (as defined at 40 CFR 35.2005(20))

- Uncontaminated pumped groundwater

- Discharge from potable water sources

- Foundation drains

- Air conditioning condensation

- Irrigation water, springs

- Water from crawl space pumps

- Footing drains

- Lawn watering

- Individual resident car washing

- De-chlorinated swimming pool discharges

- Street wash waters

- Residential building wash waters without detergents

If these discharges are identified as significant contributors to the MS4, they must be considered an “illicit discharge” and addressed in the IDDE Plan (i.e., control these sources so they are no longer significant contributors of pollutants, and/or eliminate them entirely).

# Figure 1-1. IDDE Investigation Procedure Framework

# Table 1-1. IDDE Program Implementation Timeline

| IDDE Program Requirement | Completion Date from Effective Date of Permit |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1 Year | 1.5 Years | 2 Years | 3 Years | 7 Years | 10 Years |
| Written IDDE Program Plan | X |  |  |  |  |  |
| SSO Inventory | X |  |  |  |  |  |
| Initial Outfall Ranking | X |  |  |  |  |  |
| Written Catchment Investigation Procedure |  | X |  |  |  |  |
| Phase I Mapping |  |  | X |  |  |  |
| Phase II Mapping |  |  |  |  |  | X |
| IDDE Regulatory Mechanism or By-law (if not already in place) |  |  |  | X |  |  |
| Dry Weather Outfall Screening |  |  |  | X |  |  |
| Follow-up Ranking of Outfalls and Interconnections |  |  |  | X |  |  |
| Catchment Investigations – Problem Outfalls |  |  |  |  | X |  |
| Catchment Investigations – all Problem, High and Low Priority Outfalls |  |  |  |  |  | X |

Effective date of the permit is July 1, 2018

# Authority and Statement of IDDE Responsibilities

## Legal Authority

Instructions: Use the following language if your municipality already has an illicit discharge bylaw, ordinance, or other regulatory mechanism in place, as required by the 2003 MS4 Permit.

The ##MUNICIPALITY has adopted a ##NAME OF BYLAW (##REVISION DATE with adequate legal authority to:

- Prohibit illicit discharges

- Investigate suspected illicit discharges

- Eliminate illicit discharges, including discharges from properties not owned by or controlled by the MS4 that discharge into the MS4 system

- Implement appropriate enforcement procedures and actions.

A  copy of the bylaw is included in Appendix A.  The ##MUNICIPALITY will review its current ##NAME OF BYLAW and related land use regulations and policies for consistency with the 2017 MS4 Permit.

Instructions: Use the following language if your municipality has not adopted an illicit discharge bylaw, ordinance, or other regulatory mechanism in place, as required by the 2003 MS4 Permit.

The ##MUNICIPALITY will adopt a bylaw, ordinance, or other regulatory mechanism to provide the ##MUNICIPALITY with adequate legal authority to:

- Prohibit illicit discharges

- Investigate suspected illicit discharges

- Eliminate illicit discharges, including discharges from properties not owned by or controlled by the MS4 that discharge into the MS4 system

- Implement appropriate enforcement procedures and actions.

The bylaw, ordinance, or other regulatory mechanism will meet the requirements of the 2017 MS4 Permit and will be in place as soon as possible.  New Permittees not covered under the previous permit the bylaw, ordinance, or other regulatory mechanism shall be in place within 3 years of the permit effective date (July 1, 2021).

## Statement of Responsibilities

The ##AGENCY OR DEPARTMENT is the lead municipal agency or department responsible for implementing the IDDE program pursuant to the provisions of the ##NAME OF BYLAW. Other agencies or departments with responsibility for aspects of the program include:

Instructions: List other municipal agencies or departments with responsibility for aspects of the IDDE program.  Describe the process for coordination and data sharing between these agencies and departments.

- Department of Public Works - ##RESPONSIBILITIES

- Highway Department - ##RESPONSIBILITIES

- Sewer Department - ##RESPONSIBILITIES

- Building Inspector and/or Code Enforcement Officer - ##RESPONSIBILITIES

- Licensed Plumbing Inspector - ##RESPONSIBILITIES

- Health Department - ##RESPONSIBILITIES

- Engineering Department - ##RESPONSIBILITIES

- Conservation Agent - ##RESPONSIBILITIES

- Conservation Commission - ##RESPONSIBILITIES

- Planning Board Chairperson - ##RESPONSIBILITIES

- Board of Selectmen - ##RESPONSIBILITIES

- Town Administrator and/or Mayor - ##RESPONSIBILITIES

# Stormwater System Mapping

A copy of the existing storm system map is provided in Appendix B.

The MS4 Permit requires the storm system map to be updated in two phases as outlined below. The ##AGENCY OR DEPARTMENT is responsible for updating the stormwater system mapping pursuant to the 2017 MS4 Permit. The ##MUNICIPALITY will report on the progress towards completion of the storm system map in each annual report. Updates to the stormwater mapping will be included in Appendix B.

## Phase I Mapping

Phase I mapping must be completed within two (2) years of the effective date of the permit (July 1, 2020) and include the information per Part 2.3.4.5.a of the MS4 Permit.

- Outfalls and receiving waters (previously required by the MS4-2003 permit)

- Open channel conveyances (swales, ditches, etc.)

- Interconnections with other MS4s and other storm sewer systems

- Municipally owned stormwater treatment structures

- Water bodies identified by name and indication of all use impairments as identified on the most recent EPA approved Massachusetts Integrated List of Waters report

- Initial catchment delineations. Topographic contours and drainage system information may be used to produce initial catchment delineations.

## Phase II Mapping

Phase II mapping must be completed within ten (10) years of the effective date of the permit (July 1, 2028) and include the information per Part 2.3.4.5.b of the MS4 Permit.

- Outfall spatial location (latitude and longitude with a minimum accuracy of +/-30 feet)

- Pipes

- Manholes

- Catch basins

- Refined catchment delineations. Catchment delineations must be updated to reflect information collected during catchment investigations.

- Municipal Sanitary Sewer system (if available)

- Municipal combined sewer system (if applicable).

# Sanitary Sewer Overflows (SSOs)

Instructions: If a community does not have any municipal sewer note so here and remove all of the rest of this section.

##MUNICIPALITY has no municipally owned sewer and therefore no Sanitary Sewer Overflows (SSOs).

Instructions: If a community does not have any SSO’s, note so here and remove the rest of this section.

##MUNICIPALITY has no Sanitary Sewer Overflows (SSOs).

Discharges of wastewater from any point sources, including sanitary sewer overflows (SSO’s) shall be reported in accordance with Part II, Section D.1.e. of the General Requirements of the Publicly Owned Treatment Works General Permit.

Instructions: Use the following language if your municipality has not adopted an illicit discharge bylaw, ordinance, or other regulatory mechanism in place, as required by the 2003 MS4 Permit.

The MS4 Permit requires municipalities to prohibit illicit discharges, including sanitary sewer overflows (SSOs), to the separate storm sewer system. SSOs are discharges of untreated sanitary wastewater from a municipal sanitary sewer that can contaminate surface waters, cause serious water quality problems and property damage, and threaten public health. SSOs can be caused by blockages, line breaks, sewer defects that allow stormwater and groundwater to overload the system, power failures, improper sewer design, and vandalism.

The ##MUNICIPALITY has completed an inventory of SSOs that have discharged to the MS4 within the five (5) years prior to the effective date of the 2017 MS4 Permit, based on review of available documentation pertaining to SSOs (Appendix B). The inventory includes all SSOs that occurred during wet or dry weather resulting from inadequate conveyance capacities or where interconnectivity of the storm and sanitary sewer infrastructure allows for transfer of flow between systems.

Discharges of wastewater from any point sources, including sanitary sewer overflows (SSO’s) shall be reported in accordance with Part II, Section D.1.e. of the General Requirements of the Publicly Owned Treatment Works General Permit (Appendix B).

The inventory in Appendix B will be updated by the ##AGENCY OR DEPARTMENT when new SSOs are detected. The SSO inventory will be included in the annual report, including the status of mitigation and corrective measures to address each identified SSO.

# Assessment and Priority Ranking of Outfalls

The MS4 Permit requires an assessment and priority ranking of outfalls in terms of their potential to have illicit discharges related public health significance. The ranking helps determine the priority order for performing IDDE investigations and meeting permit milestones.

## Outfall Catchment Delineations

The catchments for each of the MS4 outfalls will be delineated to define contributing areas for investigation of potential sources of illicit discharges.  Initial catchment delineations will be completed as part of the Phase I mapping, and refined catchment delineations will be completed as part of the Phase II mapping to reflect information collected during catchment investigations

## Outfall and Interconnection Inventory and Initial Ranking

The ##AGENCY OR DEPARTMENT will complete an initial outfall and interconnection inventory and priority ranking to assess illicit discharge potential based on existing information. The initial inventory and ranking will be completed within one (1) year from the effective date of the permit. An updated inventory and ranking will be provided in each annual report thereafter. The inventory will be updated annually to include data collected in connection with dry weather screening and other relevant inspections.

Outfalls and interconnections will be classified into one of the following categories:

- Excluded outfalls:

- Outfalls/interconnections with no potential for illicit discharges including roadway drainage in undeveloped areas with no dwellings and no sanitary sewers; drainage for athletic fields, parks or undeveloped green space and associated parking without services; cross-country drainage alignments (that neither cross nor are in proximity to sanitary sewer alignments) through undeveloped land.

- Problem Outfalls: Outfalls/interconnections with known or suspected contributions of illicit discharges based on existing information shall be designated as Problem Outfalls. This shall include any outfalls/interconnections where previous screening indicates likely sewer input. Problem Outfalls need not be screened pursuant to Dry Weather Outfall and Interconnection Screening and Sampling.  Likely sewer input indicators are any of the following:

- Olfactory or visual evidence of sewage,

- Ammonia ≥ 0.5 mg/L, surfactants ≥ 0.25 mg/L, and bacteria levels greater than the water quality criteria applicable to the receiving water, or

- Ammonia ≥ 0.5 mg/L, surfactants ≥ 0.25 mg/L, and detectable levels of chlorine.

- High Priority Outfalls: Outfalls/interconnections that have not been classified as Problem Outfalls and that are:

  - Discharging to an area of concern to public health due to proximity of public beaches, recreational areas, drinking water supplies or shellfish beds

  - Determined by the permittee as high priority based on the characteristics listed in Appendix C .

- Low Priority Outfalls: Outfalls/interconnections determined by the permittee as low priority based on the characteristics listed below or other available information.

Outfalls will be ranked into the above priority categories (except for excluded outfalls, which may be excluded from the IDDE program) based on the following characteristics of the defined initial catchment areas, where information is available. To prioritize initial mapping and outfall assessment work the permittee is using location-specific characteristics of water body impairments to focus initial work as included in Appendix B For the initial outfall ranking and catchment investigations this approach will target the worst areas first.

Instructions: Delete the following characteristics where data is not available or that do not apply, and list and/or describe other relevant characteristics used to initially rank outfalls

- Previous screening results – previous screening/sampling results indicate likely sewer input (see criteria above for Problem Outfalls).

- Past discharge complaints and reports.

- Poor receiving water quality – the following guidelines are recommended to identify waters as having a high illicit discharge potential:

  - Exceeding water quality standards for bacteria

  - Ammonia levels above 0.5 mg/l

  - Surfactants levels greater than or equal to 0.25 mg/l

- Density of generating sites – Generating sites are those places, including institutional, municipal, commercial, or industrial sites, with a potential to generate pollutants that could contribute to illicit discharges. Examples of these sites include, but are not limited to, car dealers; car washes; gas stations; garden centers; and industrial manufacturing areas.

- Age of development and infrastructure – Industrial areas greater than 40 years old and areas where the sanitary sewer system is more than 40 years old will probably have a high illicit discharge potential. Developments 20 years or younger will probably have a low illicit discharge potential.

- Sewer conversion – Contributing catchment areas that were once serviced by septic systems, but have been converted to sewer connections may have a high illicit discharge potential.

- Historic combined sewer systems – Contributing areas that were once serviced by a combined sewer system, but have been separated may have a high illicit discharge potential.

- Surrounding density of aging septic systems – Septic systems thirty years or older in residential land use areas are prone to have failures and may have a high illicit discharge potential.

- Culverted streams – Any river or stream that is culverted for distances greater than a simple roadway crossing may have a high illicit discharge potential.

- Water quality limited waterbodies that receive a discharge from the MS4 or waters with approved TMDLs applicable to the permittee, where illicit discharges have the potential to contain the pollutant identified as the cause of the water quality impairment.

The following is an initial outfall prioritization flowchart, see Appendix C for an outfall inventory and priority ranking matrix:

Is the waterbody impaired and listed in Part II Summary of Receiving Waters in the NOI?

No:  Excluded from priority ranking

Yes: Does Outfall/Interconnection have no potential for illicit discharges (roadway drainage in undeveloped areas with no dwellings and no sanitary sewers/septic within 100’; drainage for athletic fields, parks or undeveloped green space and associated parking without services; cross-country drainage alignments that neither cross nor are in proximity to sanitary sewer alignments through undeveloped land)

(Part 2.3.4.7.a.ii)

No: Include outfall inventory and priority ranking matrix

Yes:  Excluded from priority ranking

# Dry Weather Outfall Screening and Sampling

Dry weather flow is a common indicator of potential illicit connections. The MS4 Permit requires all outfalls/interconnections (excluding Problem and Excluded Outfalls) to be inspected for the presence of dry weather flow. The ##AGENCY OR DEPARTMENT is responsible for conducting dry weather outfall screening, starting with High Priority outfalls, followed by Low Priority outfalls, based on the initial priority rankings described in the previous section by the end of Year 3.

Dry weather outfall Screening and Sampling shall be completed in accordance with Part 2.3.4.7.b of the MS4 Permit.  Plans and procedures for such screening and sampling shall be incorporated into this plan.

  - Dry Weather Screening/Sampling Procedure

Instructions: The dry weather screening and sampling procedures described in this section are based on the requirements outlined in the 2017 MS4 Permit.

Municipalities should include example Sample Labels, Field Sheets and Chain of Custody forms in the appendices of the IDDE Plan.

    - General Procedure

The dry weather outfall inspection and sampling procedure consists of the following general steps:

- Identify outfall(s) to be screened/sampled based on initial outfall inventory and priority ranking

- Acquire the necessary staff, mapping, and field equipment (see Table 6-1 for list of potential field equipment)

- Conduct the outfall inspection during dry weather:

  - Mark and photograph the outfall

  - Record the inspection information and outfall characteristics (using paper forms or digital form using a tablet or similar device) (see form in Appendix D)

  - Look for and record visual/olfactory evidence of pollutants in flowing outfalls including odor, color, turbidity, and floatable matter (suds, bubbles, excrement, toilet paper or sanitary products). Also observe outfalls for deposits and stains, vegetation, and damage to outfall structures.

- If flow is observed, sample and test the flow following the procedures described in the following sections.

- If no flow is observed, but evidence of illicit flow exists (illicit discharges are often intermittent or transitory), revisit the outfall during dry weather within one week of the initial observation, if practicable, to perform a second dry weather screening and sample any observed flow. Other techniques can be used to detect intermittent or transitory flows including conducting inspections during evenings or weekends and using optical brighteners.

- Input results from screening and sampling into spreadsheet/database. Include pertinent information in the outfall/interconnection inventory and priority ranking.

- Include all screening data in the annual report.

    - Field Equipment

Table 6-1. Field Equipment – Dry Weather Outfall Screening and Sampling

| Equipment | Use/Notes |
| --- | --- |
| Clipboard | For organization of field sheets and writing surface |
| Field Sheets | Field sheets for both dry weather inspection and Dry weather sampling should be available with extras |
| Chain of Custody Forms | To ensure proper handling of all samples |
| Pens/Pencils/Permanent Markers | For proper labeling |
| Nitrile Gloves | To protect the sampler as well as the sample from contamination |
| Flashlight/headlamp w/batteries | For looking in outfalls or manholes, helpful in early mornings as well |
| Cooler with Ice | For transporting samples to the laboratory |
| Digital Camera | For documenting field conditions at time of inspection |
| Personal Protective Equipment (PPE) | Reflective vest, Safety glasses and boots at a minimum |
| GPS Receiver | For taking spatial location data |
| Water Quality Sonde | If needed, for sampling conductivity, temperature, pH |
| Water Quality Meter | Hand held meter, if available, for testing for various water quality parameters such as ammonia, surfactants and chlorine |
| Test Kits | Have extra kits on hand to sample more outfalls than are anticipated to be screened in a single day |
| Label Tape | For labeling sample containers |
| Sample Containers | Make sure all sample containers are clean. Keep extra sample containers on hand at all times. Make sure there are proper sample containers for what is being sampled for (i.e., bacteria requires sterile containers). |
| Pry Bar or Pick | For opening catch basins and manholes when necessary |
| Sandbags | For damming low flows in order to take samples |
| Small Mallet or Hammer | Helping to free stuck manhole and catch basin covers |
| Utility Knife | Multiple uses |
| Measuring Tape | Measuring distances and depth of flow |
| Safety Cones | Safety |
| Hand Sanitizer | Disinfectant/decontaminant |
| Zip Ties/Duct Tape | For making field repairs |
| Rubber Boots/Waders | For accessing shallow streams/areas |
| Sampling Pole/Dipper/Sampling Cage | For accessing hard to reach outfalls and manholes |

    - Sample Collection and Analysis

If flow is present during a dry weather outfall inspection, a sample will be collected and analyzed for the required permit parameters. The general procedure for collection of outfall samples is as follows:

- Fill out all sample information on sample bottles and field sheets (see Appendix D for Sample Labels and Field Sheets)

- Put on protective gloves (nitrile/latex/other) before sampling

- Collect sample with dipper or directly in sample containers. If possible, collect water from the flow directly in the sample bottle. Be careful not to disturb sediments.

- If using a dipper or other device, triple rinse the device with distilled water and then in water to be sampled (not for bacteria sampling)

- Use test strips, test kits, and field meters (rinse similar to dipper) for most parameters (see Table 6-1)

- Place laboratory samples on ice for analysis of bacteria and pollutants of concern

- Fill out chain-of-custody form (Appendix D) for laboratory samples

- Deliver samples to ##NAME OF LABORATORY(s)

- Dispose of used test strips and test kit ampules properly

- Decontaminate all testing personnel and equipment

In the event that an outfall is submerged, either partially or completely, or inaccessible, field staff will proceed to the first accessible upstream manhole or structure for the observation and sampling and report the location with the screening results. Field staff will continue to the next upstream structure until there is no longer an influence from the receiving water on the visual inspection or sampling.

Field test kits or field instrumentation are permitted for all parameters except indicator bacteria and any pollutants of concern. Field kits need to have appropriate detection limits and ranges.

  - Follow-up Ranking of Outfalls and Interconnections

The ##MUNICIPALITY will update and re-prioritize the initial outfall and interconnection rankings based on information gathered during dry weather screening. The rankings will be updated periodically as dry weather screening information becomes available, but will be completed within three (3) years of the effective date of the permit (July 1, 2021).

Outfalls/interconnections where relevant information was found indicating sewer input to the MS4 or sampling results indicating sewer input are highly likely to contain illicit discharges from sanitary sources.

Such outfalls/interconnections will be ranked at the top of the High Priority Outfalls category for investigation. Other outfalls and interconnections may be re-ranked based on any new information from the dry weather screening

# Catchment Investigations

Once stormwater outfalls with evidence of illicit discharges have been identified, various methods can be used to trace the source of the potential discharge within the outfall catchment area. Catchment investigation techniques include but are not limited to review of maps, historic plans, and records; manhole observation; dry and wet weather sampling; video inspection; smoke testing; and dye testing.

Catchment Investigations shall be completed in accordance with Part 2.3.4.8 of the MS4 Permit.  A written catchment investigation procedure shall be developed and incorporated into this plan within 18 months of the permit effective date.  Investigations of catchments associated with Problem Outfalls shall begin no later than two (2) years from the permit effective date and shall be completed within seven (7) years.

## Illicit Discharge Removal

When the specific source of an illicit discharge is identified, the ##MUNICIPALITY will exercise its authority as necessary to require its removal. The annual report will include the status of IDDE investigation and removal activities including the following information for each confirmed source:

- The location of the discharge and its source(s)

- A description of the discharge

- The method of discovery

- Date of discovery

- Date of elimination, mitigation or enforcement action OR planned corrective measures and a schedule for completing the illicit discharge removal

- Estimate of the volume of flow removed.

# Training

Annual IDDE training will be made available to employees involved in the IDDE program. This training will at a minimum include information on how to identify illicit discharges and SSOs and may also include additional training specific to the functions of particular personnel and their function within the framework of the IDDE program. Training records will be maintained in Appendix F. The frequency and type of training will be included in the annual report.

# Progress Reporting

The progress and success of the IDDE program will be evaluated on an annual basis. The evaluation will be documented in the annual report and will include the following indicators of program progress:

- Number of SSOs and illicit discharges identified and removed

- Number and percent of total outfall catchments served by the MS4 evaluated using the catchment investigation procedure

- Number of dry weather outfall inspections/screenings

- Number of wet weather outfall inspections/sampling events

- All dry weather and wet weather screening and sampling results

- Estimate of the volume of sewage removed, as applicable

- Number of employees trained annually.

The success of the IDDE program will be measured by the IDDE activities completed within the required permit timelines.

Appendix A

Legal Authority (IDDE Bylaw or Ordinance)

Appendix B

List of Impaired Waters

Storm System Mapping

& SSO Inventory

Instructions: Obtain the List of Impaired Waters from you NOI.  Copy and paste in this appendix.

If you have an existing SSO Inventory, also copy and paste in this appendix.

EXAMPLES ONLY HAVE BEEN INCLUDED IN THIS APPENDIX;  THESE HAVE NOT BEEN FINALIZED BUT ARE ONLY EXAMPLES TO BE FURTHER EVALUATED AND THE SPECIFIC STANDARD OPERATING PROCEDURES AND FORMS FOR NEW HAMPSHIRE COMMUNITIES WILL BE DEVELOPED PER THE APPROVED SCHEDULE.

# LIST OF IMPAIRED WATERS

# (Copy the information directly from your NOI OR fill out the table below)

# ##MUNICIPALITY, New Hampshire

| Water Body Name | Segment ID | Category | Impairment(s) | Associated Approved TMDL |
| --- | --- | --- | --- | --- |

Category 4a Waters – impaired water bodies with a completed Total Maximum Daily Load (TMDL).

Category 4c Waters – impaired water bodies where the impairment is not caused by a pollutant. No TMDL required.

Category 5 Waters – impaired water bodies that require a TMDL.

“Approved TMDLs” are those that have been approved by EPA as of the date of issuance of the 2016 MS4 Permit.

# Table 9-1. SSO Inventory

##MUNICIPALITY, New Hampshire

Revision Date: ##DATE OF LAST UPDATE

| SSO Location1 | Discharge Statement2 | Date3 | Time Start3 | Time End3 | Estimated Volume4 | Description5 | Mitigation Completed6 | Mitigation Planned7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

1 Location (approximate street crossing/address and receiving water, if any)

2 A clear statement of whether the discharge entered a surface water directly or entered the MS4

3 Date(s) and time(s) of each known SSO occurrence (i.e., beginning and end of any known discharge)

4 Estimated volume(s) of the occurrence

5 Description of the occurrence indicating known or suspected cause(s)

6 Mitigation and corrective measures completed with dates implemented

7 Mitigation and corrective measures planned with implementation schedules

Instructions: Complete this SSO inventory for all Sanitary Sewer Overflows (SSOs) that have discharged to the MS4 within the five (5) years prior to the effective date of the 2017 MS4 Permit. This inventory must be completed within one (1) year of the effective date of the permit. The table should be updated with information on new SSOs that are detected.

Appendix C

Outfall Inventory and

Priority Ranking Matrix

Instructions:  These are the results from your mapping and is specific to each community. Two example sheets have been included.

Appendix D

Field Forms,

Sample Bottle Labels, and

Chain of Custody Forms

Appendix to include copies of the following field sampling documents once fully developed in accordance with the 2017 MS4 Permit:

- Dry weather outfall inspection/sampling form

- Wet weather outfall inspection/sampling form

- Manhole inspection form

- Example sample labels (provided by laboratory)

- Example chain-of-custody form(s) (provided by laboratory(s))

EXAMPLES ONLY HAVE BEEN INCLUDED IN THIS APPENDIX; THESE HAVE NOT BEEN FINALIZED BUT ARE ONLY EXAMPLES TO BE FURTHER EVALUATED AND THE SPECIFIC STANDARD OPPERATING PROCEDURES & FORMS FOR NEW HAMPSHIRE COMMUNITIES WILL BE DEVELOPED PER THE APPROVED SCHEDULE.

Appendix E

Water Quality Analysis Instructions,

User’s Manuals and

Standard Operating Procedures

Appendix to include copies of water quality analysis instructions, procedures, and SOPs for all sample parameters and all meters or field test kits that are used for analysis once fully developed in accordance with the 2017 MS4 Permit. This includes the manufacturer’s instructions for how to use field test kits as well as the manufacturer’s instructions or user’s manual for any field instrumentation.

EXAMPLES ONLY HAVE BEEN INCLUDED IN THIS APPENDIX; THESE HAVE NOT BEEN FINALIZED BUT ARE ONLY EXAMPLES TO BE FURTHER EVALUATED AND THE SPECIFIC STANDARD OPPERATING PROCEDURES & FORMS FOR NEW HAMPSHIRE COMMUNITIES WILL BE DEVELOPED PER THE APPROVED SCHEDULE.

Appendix F

IDDE Employee Training Record

Illicit Discharge Detection and Elimination (IDDE)

Employee Training Record

##MUNICIPALITY

| Date | Type of Training | Participants |
| --- | --- | --- |

Appendix G

Source Isolation and Confirmation Methods:

Instructions, Manuals, and SOPs

Appendix to provide manufacturer instructions, manuals and procedures and any in-house SOPs used to perform source isolation and confirmation for illicit discharges once fully developed in accordance with the 2017 MS4 Permit.

EXAMPLES ONLY HAVE BEEN INCLUDED IN THIS APPENDIX; THESE HAVE NOT BEEN FINALIZED BUT ARE ONLY EXAMPLES TO BE FURTHER EVALUATED AND THE SPECIFIC STANDARD OPPERATING PROCEDURES & FORMS FOR NEW HAMPSHIRE COMMUNITIES WILL BE DEVELOPED PER THE APPROVED SCHEDULE.